K. BIHICHE, M. LAMSAADI, M. NAÏMI, H.ELHARFI, M. KADDIRI, A. LOUARAYCHI

Thermosolutal bifurcation phenomena in a rectangular enclosure filled with non –Newtonian fluids

Sultan Moulay Slimane University, Faculty of  Sciences and Technologies, Physics Department, Laboratory of Flows and Transfers Modelling (LAMET), B.P.523, Beni-Mellal 23000, Morocco

khadija.bihiche@gmail.com

Abstract- This paper reports an analytical and numerical study of double-diffusive natural convection in a non-Newtonian power-law fluid contained in a horizontal rectangular enclosure submitted to uniform heat and mass fluxes along its short vertical sides, while the horizontal ones are insulated and impermeable. An analytical model, based on the parallel flow approximation, is proposed for the case of a shallow layer (A>>1). The particular case where the thermal buoyancy forces are dominating the flow (N = -0.98) is considered. For this situation the two saddle-nodes bifurcation are predicted. The study is completed by a numerical solution of the full governing equations.

# Keywords : double-diffusive natural convection, finite volume method, bifurcation phenomena, non-Newtonian fluids.

# INTRODUCTION

The study of double diffusive natural convection simultaneously driven by temperature and concentration gradients in a fluid-saturated rectangular cavity has received considerable attention in recent years due to its wide variety of applications in science and technology such as in solar energy collection, nuclear reactor insulation, the cooling of electronic devices, drying technologies, energy storage and chemical vapor deposition processes used for the manufacture of semiconductor devices, etc.

A brief review of some important studies related to double-diffusive natural convection in an enclosure is outlined below. The effect of variable density on thermosolutal natural convection in a binary mixture of ideal gasses contained in a vertical enclosure was numerically investigated by Sun et al. [1]. The results showed that the effects of non-Boussinesq were more significant in the opposing flow than in the aiding flow where both solutal and thermal forces are equal in intensity. Steady conjugate double-diffusive natural convective heat and mass transfer in a two-dimensional variable porosity layer sandwiched between two walls has been studied numerically by Al-Farhany and Turan [2]. Bera et al. [3] have presented a numerical study to understand the influence of local thermal non-equilibrium (LTNE) state on double-diffusive natural convection in a square cavity filled with fluid-saturated porous medium. The results indicate that the impact of LTNE state on the heat transfer rate and thermal distribution is significant but it is negligible on the mass transfer rate and solute distribution. Harzallah et al. [4] have numerically studied the effects of finite thickness conducting walls as well as the mechanical anisotropy on the unsteady double-diffusive natural convection taking place in a porous material sandwiched between two-vertical finite thickness walls maintained at constant but different temperatures. The two-vertical left and right interfaces wall-porous material are, respectively, kept at constant high and low concentration values. The remaining two-horizontal walls are assumed to be impermeable to mass transfer and thermally insulated. Steady heat and mass transfer by natural convection flow of a heat generating fluid inside a rectangular enclosure in the presence of a transverse magnetic field was studied numerically by M. Teamah [5]. It has been reported that the heat and mass transfer mechanisms and the flow characteristics inside the enclosure depended strongly on the strength of the magnetic field and heat generation or absorption effects. The magnetic field was found to reduce the heat transfer and fluid circulation within the enclosure. Later, Teamah et al. [6] analyzed  the double-diffusive natural convective flow in an inclined rectangular cavity in the presence of magnetic field and heat source. They found that the inclination angle had a considerable influence on heat and mass transfer rate so that the maximum average Nusselt and Sherwood numbers were obtained at two inclination angles, γ = 45° and 135°.

Makayssi et al. [7] performed both analytical and numerical analyses of double-diffusive natural convection in a non-Newtonian power-law fluid contained in a horizontal rectangular enclosure. The authors examined the influence of different parameters (aspect ratio, Lewis number, Buoyancy ratio number, power-law behavior index, Prandtl number, thermal Rayleigh number) on the fluid flow and heat and mass transfer characteristics. Natural convection through porous cavity filled with a non-Newtonian binary fluid was studied by Ben khelifa et al [8], Criteria, for supercritical and subcritical onset of motions, are predicted. Oueslati et al. [9] have numerically investigated double-diffusive natural convection with entropy generation in an enclosure partially heated and salted from the left vertical sidewall. They concentrated their study on influences of the major parameters (Rayleigh number, buoyancy ratio, source length, Lewis number and source location) on the rate of heat and mass transfer in the rectangular cavity with an aspect ratio (Ar = 4).

The aim of the present investigation is to study the bifurcation character on natural double-diffusive convection problem inside a two-dimensional horizontal rectangular enclosure filled with a non-Newtonian fluid.

# Mathematical formulation

The studied configuration, sketched in Fig. 1, is a rectangular enclosure of height  and lengthwith the long horizontal rigid walls insulated and impermeable and the short vertical ones submitted to constant heat and mass densities of fluxes, and, respectively. In the formulation of the present problem, the following assumptions are made: The fluid is incompressible, the flow of a non-Newtonian fluid through the cavity is governed by the power law, The viscous dissipation is negligible.  the flow is laminar, The third dimension of the cavity is such that the problem can be considered as two-dimensional,the Soret and Dufour effect are negligible, the properties of the fluid and the porous medium are constants and Boussinesq approximations are valid.

y’, v’

Control volum

q’

q’

H’

j’

j’

L’

x’, u’

Fig. 1: Schematic diagram of the problem and coordinate system

The dimensionless governing equations describing conservation of mass, momentum, and energy, written in terms of velocity components, pressure, temperature and concentration are given as:

(1)

(2)

(3)

(4)                                                                     (5)

The dimensionless boundary conditions, for the physical system considered here, are

(6)

(7)

The above equations let appears some dimensionless parameters that govern the problem, namely, the aspect ratio of the enclosure, A, the Lewis number, Le, the buoyancy ratio, N, the generalized Prandtl, Pr, and thermal Rayleigh, RaT, numbers, whose expressions are

(8)

# Numerics

Equations (1) - (5) associated with Equations (6)-(7) have been solved by using a finite volume method and SIMPLER algorithm in a staggered uniform grid system [10]. The convergence has been considered as reached when  , where stands for the value of u, v, p,T or S at the kth iteration level and grid location (i, j) in the plane (x, y).

The mesh size has been chosen so that a best compromise between running time and accuracy of the results may be found. The procedure has been based on grid refinement until the numerical results agree, within reasonable accuracy, with the analytical ones, obtained from the parallel flow approach developed in the next section. Numerical tests have been performed to determine the minimum aspect ratio above which the flow can be assumed to be parallel. In the range of the parameters considered in this investigation it was found that the numerical results can be considered independent of the aspect ratio when. For this reason most of the numerical results reported here were obtained for A = 24 with typically 380×80 mesh points. Typical numerical results are presented in figures 3(a)-(b), on the graphs streamlines, isotherms and

isoconcentrates are presented from top to bottom. The results clearly illustrate the fact that for a shallow cavity  the flow in the core region of the enclosure is essentially parallel while the temperature and concentration in the core are linearly stratified in the horizontal direction. The analytical solution, developed in the following section, will rely on these observations.

# Analytical solution

The results presented in Fig. 3(a)-(b), allow the following appropriate simpliﬁcations:

(9)

where CT and CS are unknown constant temperature and concentration gradients in the x-direction. Using the above approximations, Eqs. (1)–(5) reduce to the following systems of equations

(10)

(11)

With

(12)

(13)

as return flow, mean temperature and concentration conditions, respectively.

The integration of Eqs. (10) and (11), coupled with the conditions (12) and (13), leads to analytical expressions of velocity, temperature and concentration.     However, such an operation is difficult to carry out owing to the particular nature of the governing equations and requires, therefore, a special numerical treatment [11].

# RESULTS AND DISCUSSION

The fact of imposing uniform heat and mass fluxes, as boundary conditions, leads to flow characteristics independent on the aspect ratio, A, when this parameter is large enough. . The approximate solution, developed in the preceding section, on the basis of the parallel flow assumption, is thus valid asymptotically in the limit of a shallow cavity (A >>1). The analytical solution, displayed by a solid line, is seen to be in

excellent agreement with the numerical solution of the full governing equations, depicted by black dots.

In Fig. 2 the stream function amplitude  is plotted as a function of  for Le = 10, N = -0.98 and -1 and various value of n. As expected, the results indicate that the convective flow increases monotonously with. For low values of   it is observed that for the case N = − 0.98 convection is weak and can be seen as a degeneracy of the conductive solution obtained for N = −1. This type of flow is maintained up to a critical Rayleigh number ( for n = 0.6,  for n = 1 and  for n = 1.4) where a first saddle-node bifurcation occurs. This intermediate branch (shown as a dotted line) is connected to another saddle-node bifurcation occurring at a critical Rayleigh number ( for n = 0.6,  for n = 1 and  for n = 1.4). Then, upon increasing the Rayleigh number the results obtained for N = −0.98 and −1 are approximately the same. In the region between for n = 0.6,  for n = 1 and three solutions are possible for a given Rayleigh number.

- Fig. 2: Bifurcation diagram for Le = 10, N = -1 and N = -0.98 and various value of n ((a) n=0.6, (b) n=1.0, (c) n=1.4)

It is also observed from this figures that the fluid flow seem to be rather sensitive to the flow behavior index, n. Thus, compared to Newtonian case (n = 1), the shear-thinning behavior (0 < n < 1) enhances the fluid circulation while the shear-thickening one (n > 1) produces an opposite effect. Recently, analogous observations were reported by Lamsaadi et al. [11] in their numerical and analytical investigations related to pure thermal convection in the same geometrical configuration.

The existence of multiple solutions in these ranges was demonstrated numerically for the case, Le = 10 and N = -0.98. Fig. 3(a) shows the results obtained by using the convective flow   as initial conditions for the numerical code. The resulting flow in the core of the layer is observed to be parallel while the temperature and concentration are linearly stratified in the horizontal direction.  Starting the numerical solution with the rest state and uniform temperature and concentration yields the results depicted in Fig. 3(b). The strength of flow circulation is found to be considerably weaker as indicated by the temperature and concentration fields which are now almost purely diffusive. The excellent agreement between the analytical and the numerical solutions is noticed.

n = 0.6

n = 1.0

n = 1.4

--- (a) ---

n = 0.6

- n = 1.0

- n = 1.4

- --- (b) ---

Fig. 3: Streamlines (left), isotherms (middle) and isoconcentrationss (right) for, Le = 10, N = -0.98 and various values of n

# Conclusion

The present paper is devoted to numerical and analytical experiments on natural double-diffusive convection in a two-dimensional horizontal shallow enclosure (A>>1), filled with non-Newtonian power-law fluids, in the case where both short vertical sides are submitted to uniform heat and mass fluxes (Neumann type condition) while the horizontal boundaries are insulated and impermeable. The summary of the major results is:

(1) An analytical solution, based on the parallel flow approximation, has been derived for the case of an infinite layer (A = 24).  Solving analytically the full set of governing equations is by far a much easier task than numerical simulations.

(2) For  N = −0,98, the thermal buoyancy forces are clearly dominating the flow , for this situation two  saddle-nodes bifurcation occurs at two different critical Rayleigh numbers. The existence of multiple solutions, for a given set of the governing parameters, in the regions between the first and second saddle-node bifurcation, is demonstrated both analytically and numerically.

- References

[1]   H. Sun, G. Lauriat, D.L. Sun, W.Q. Tao. Transient double-diffusive

convection in an enclosure with large density variations. 2010. 53.

p. 615–625.

[2]   K. Al-Farhany, A. Turan. Non-Darcy effects on conjugate double-

diffusive natural convection in a variable porous layer sandwiched by

finite thickness walls. 2011.  54.  p. 2868–2879.

[3]   P. Bera, S. Pippal, A.K. Sharma.  A thermal non-equilibrium

approach on double- diffusive natural convection in a square porous-

medium  cavity. 2014. 78. p. 1080–1094.

[4]   H.S. Harzallah, A. Jbara, K. Slimi. Double-diffusive natural

convection in anisotropic porous medium bounded by finite thickness

walls: validity of local thermal equilibrium assumption .2014. 103.p.

207–231.

[5]   M.A. Teamah. Numerical simulation of double diffusive natural

convection in rectangular enclosure in the presences of magnetic field

and heat source.2008. 47. p. 237–248.

[6]   M.A. Teamah, A.F. Elsafty, M.Z. Elfeky, E.Z. El-Gazzar. Numerical

simulation of double-diffusive natural convective flow in an inclined

rectangular enclosure in the presence of magnetic field and heat

source, part A: effect of Rayleigh number and inclination angle. 2011.

50. p.  269–282.

[7]  T. Makayssi , M. Lamsaadi , M. Naïmi , M. Hasnaoui , A. Raji , A.

Bahlaoui. Natural double-diffusive convection in a shallow horizontal

rectangular cavity uniformly heated and salted from the side and filled

with non-Newtonian power-law fluids: The cooperating case. 2008.

49. p. 2016–2025.

[8]  N. Ben Khelifa , Z. Alloui, H. Beji, P. Vasseur. Natural convection in

a horizontal porous cavity ﬁlled with a non-Newtonian binary ﬂuid of

power-law type. 2012. 169–170. p. 15-25.

[9]  F. Oueslati, B. Ben-Beya, T. Lili.  Double-diffusive natural

convection and entropy generation in an enclosure of aspect ratio 4

with partial vertical heating and salting sources. 2012. 52. p. 605–625.

[10] S. Patankar. Num. Heat Transfer and Fluid Flow. Hemisphere. New York. 1980.

[11] M. Lamsaadi, M. Naimi,M. Hasnaoui. Natural convection heat transfer in shallow horizontal rectangular enclosures uniformly heated from the side and filled with non-Newtonian power law fluids. 2006. 47. p.  2535–2551.