I. Arroub1, A. Bahlaoui1, K. Ezzaraa1, A. Raji2, M. Hasnaoui3 and M. Naïmi2

Mixed convection heat transfer improvement inside a multiple vented cavity crossed by Al2O3-water nanofluid

1 Team of Applied Physics and New Technologies (EPANT), Polydisciplinary Faculty, Béni-Mellal, Morocco;

2 Laboratory of Flows and Transfers Modeling (LAMET), Faculty of Sciences and Technics, Béni-Mellal, Morocco;

3 Laboratory of Fluid Mechanics and Energetics (LMFE), Faculty of Sciences Semlalia, Marrakech, Morocco;

ismail.arroub@gmail.com

Abstract: The goal of this work is to study the laminar mixed convection of water-Al2O3 nanofluid in a multiple vented cavity. The effects of various controlling parameters, such as Reynolds number, 200  Re  3000, solid volume fraction of nanoparticles, 0    0.7, and mode of imposed external flow (injection and suction) on flow and thermal patterns and resulting heat transfer inside the cavity are investigated. The obtained results show that, the addition of nanoparticles leads to an improvement of heat transfer rate and an increase of mean temperature inside the enclosure. In addition, the suction mode was found to present higher heat performances and better cooling effectiveness, in comparison with the injection one.

Keywords : Numerical study, mixed convection, nanofluid, multiple vented cavity, injection, suction.

# Introduction

Mixed convection heat transfer is an important phenomenon in engineering systems due to its wide applications in electronic cooling, heat exchangers and solar collectors. Enhancement of heat transfer in such systems is very essential from the industrial and energy saving perspectives. The low thermal conductivity of conventional heat transfer fluids, such as water, is considered a primary limitation in enhancing the performance and the compactness of such thermal systems. With this aim, an innovative technique for improvement of heat transfer using nano-scale particles dispersed in a base fluid (known as nanofluid) has been extensively experimented. Consequently, the presence of the nanoparticles in the base fluids enhances the heat transfer characteristics of nanofluids [1. 2].

Some studies were focused on mixed convection flow in a lid-driven cavity. Among the recently published works, we quote that by Sebdani et al. [3] dealing with mixed convection of nanofluid in a lid-driven square cavity locally heated from below with heat source. They showed that for high Rayleigh numbers, the rate of heat transfer decreases with increase in nanoparticle volume fraction. Moreover it was found that when the heat source moved toward the cold side walls, the rate of heat transfer increases. In the case of variable heating, Abbasian et al. [4], investigated numerically mixed convection flow in a lid-driven square cavity filled with the Cu-water nanofluid with sinusoidal heating sidewalls. They proved that for a constant Grashof number, the rate of heat transfer increases with decrease in Richardson number and increase of volume fraction of nanoparticles.

Mixed convection of nanofluids in ventilated cavities has been also the object of interest during the last years. Such problems are encountered in many industrial applications such as heat exchangers, lubrication technology, chemical processing equipments, solar energy collectors and cooling of electronic equipment. Hence, in the case of applied uniform heat flux, Shahi et al. [5] conducted a numerical study of mixed convection in a partially heated square ventilated cavity, filled with copper-water nanofluid. The reported results showed an increase of heat transfer rate at the heat source surface and a decrease in the mean bulk temperature with the solid concentration. The same problem was reconsidered by Mahmoudi et al. [6] for different locations of inlet and outlet ports in the case of total constant heating. It was found that the presence of nanoparticles is more effective in Bottom-Top configuration than in the other considered configurations, while the solid concentration increase has the least effects in Top-Top arrangement.

The objective of this work is to study the effects of nanoparticles volume fraction, Reynolds number and the modes of imposed external flow on the heat transfer characteristics for steady mixed convection in a vented rectangular enclosure filled with a nanofluid.

# Problem Description and Mathematical Formulation

The schematic diagram of the geometry considered in this work is displayed in Fig. 1. It consists of a vented horizontal rectangular enclosure of length L', height H' and having an aspect ratio A = 2. The enclosure is heated from the bottom wall by a sinusoidal hot temperature (Fig. 1c) whereas the other solid boundaries are assumed to be thermally insulated. The system is cooled by an injected (Fig. 1a) or sucked (Fig. 1b) imposed flow, through the tow openings located on the lower part of the right and left vertical walls. These ports have a constant dimensionless height, B = 1/4. A third opening (B = 1/2) is localized in the middle of the upper wall in order to assure the ventilation. Thus, it is assumed that both the fluid phase and nanoparticles are in thermal equilibrium and the shape and size of solid particles are assumed to be uniform. Also, the thermo-physical properties of the nanofluid are constant except for the density variation, which is approximated by the Boussinesq model.

Therefore, using the following dimensionless variables:

A=L'H', B=h'H', x=x'H' ,y=y'H', u=u'uo', v=v'uo', t=t'uo'H', Ψ=Ψ'uo'H' T=(T'−TC')(TH'−TC'),    Ω=Ω'H'uo',     Pr=νf/αf

Ra= g βf(TH'−TC')H/αfνf,    Re=uo'H'/νf

The mixed convection governing equations using the vorticity-stream function formulation (-), can be written in the non-dimensional form as follows:

(1)

(2)

∂2Ψ∂x2+∂2Ψ∂y2=−Ω                          (3)

The dimensionless tangential and normal velocities are converted to:

u=∂Ψ∂y    ;   v=−∂Ψ∂x     and    Ω=∂v∂x−∂u∂y        (4)

The nanofluid effective density, thermal diffusivity, heat capacity and the thermal expansion coefficient are, respectively, calculated as follow:

ρnf=φρs+(1−φ)ρf                       (5)

αnf=λnf(ρcp)nf                             (6)

(ρcp)nf=φ(ρcp)s+(1−φ)(ρcp)f           (7)

(ρβ)nf=φρsβs+(1−φ)ρfβf               (8)

The effective thermal conductivity, nf, and the effective dynamic viscosity, nf, can be predicted using the following correlations produced by M. Corcione [7] using a wide number of experimental data obtained by several research groups:

|  | (9) |
| --- | --- |
|  | (10) |

Where Res is the nanoparticle Reynolds number, Tfr is the freezing point of the base liquid, ds is the diameter of the suspended nanoparticles (ds retained in this study is 100 nm) and df is the equivalent diameter of a base liquid molecule. “f”, “s” and “nf” indicate fluid, solid particles and nanofluid respectively.

II.1 Boundary Conditions

The common boundary conditions applied to the two ventilation modes would be adjusted as follows:

u = v = 0		          on the rigid walls

on the bottom wall

∂T∂n=0		         on the adiabatic walls

Where the parameter ‘a’ stands for the amplitude of the sinusoidal temperature (‘a’ is taken equal to 0.5 in this study) and ‘n’ denotes the outward normal to the adiabatic wall.

The appropriate boundary conditions associated to each mode are given in the subsequent sections

Injection case:

T = v =  = 0, u = 1 and  = y	at the left 		inlet of the cavity

T = v =  = 0, u = -1 and  = -y	at the right 		inlet of the cavity

 = B		between the left inlet port and the 		upper outlet port

 = -B		between the right inlet port and 		the upper outlet port

Suction case:

T = 0 		 at the upper inlet port

u = -1, v = 0,  = -y,  = 0  at the left outlet port

u = 1, v = 0,  = y,  = 0  at the right outlet port

 = -B		between the left outlet port and the 		upper inlet port

 = B 		between the right outlet port and 		the upper inlet port

Fig. 1: Geometry and coordinates system: a) Injection case, b) Suction case and c) Heating temperature profile.

a) Injection case

b) Suction case

c) Sinusoidal temperature profile on the heated wall

II.2 Heat Transfer

The rate of heat removal from the heating element is expressed in terms of average Nusselt number, Nu, calculated on the heated bottom wall of the cavity, which is defined by:

Nu = −1Aλnfλf0A∂T∂y|y=0dx                 (11)

# Numerical Procedure

The two-dimensional governing equations, (1)-(3), are solved by using the finite difference method with a regular mesh size. The advection terms are approximated by the second-order upwind differencing scheme which leads to a stable solution. Also, the second-order central differencing scheme is utilized to approximate the diffusive terms. The integration of the vorticity and energy equations, (1) and (2), is performed with the ADI (Alternating-Direction Implicit) method. To satisfy the mass conservation, the Poisson equation, (3), is solved by a PSOR (Point Successive Over-Relaxation) method with an optimum over-relaxation factor equal to 1.95 for the grid (201  101) retained in this work.

# Results and Discussion

In this section, the Rayleigh number was kept at a constant value Ra = 106 and the Reynolds number Re ranging between 200 and 3000. Values of these parameters involves values of the Richardson number, Ri=Ra/Re2Pr, varying in the range [1.7910-2, 4.03] which simulates natural convection, mixed convection and forced convection dominating regimes. In the following, effects of nanoparticles and modes of imposed external flow on flow patterns and thermal structures and on efficient cooling are investigated. Before dealing with this part of the results, it is to note that in the injection case, the flow is unsteady as Re is very weak. Therefore, the dynamical and thermal structures could not be presented in this situation.

Streamlines and isotherms plots illustrating the effect of Re on the dynamical and thermal fields, for the injection case, are shown in Figs. 2a-2c for both  = 0 (solid line) and  = 0.07 (dashed line - - -). For a moderate value of Re  (Re = 500), the streamlines show the presence of four closed cells [Fig. 2(a)]. More precisely, the two big cells surmounting the open lines of the forced flow, engendered by shear effects, are located in the right (clockwise cell) and left (trigonometric cell) upper corner of the cavity. While the two small closed cells, located under the open lines and near the heated wall, are due to buoyancy forces and shear effect. Indeed, the heat evacuated by the hot wall is received by the open lines, via these small cells, and then transferred to the outlet of the cavity. A very limited qualitative effect can be observed in the flow structure by increasing  from 0 to 0.07. The corresponding isotherms are generally tightened near the heated wall but especially from the inlet to the middle, indicating a good convective heat exchange, between the lower surface along its first half and the nanofluid. This behavior is in accordance with the imposed temperature profile at the bottom, which gives rise to a temperature higher than the average hot one, TH', as displayed in Fig. 1c. Also, the space above the open lines of the forced flow is at a uniform cold temperature. This aspect testifies to the absence of thermal interaction between the active wall and the others walls. The effect of  is more perceptible in the region occupied by the small cells since the isotherms corresponding to pure fluid and  = 0.07 are spaced. An increase of Re up to 1000 leads to an increase in the size and intensity of the two lower cells as a result of the increase of the effect of forced convection [Fig. 2(b)]. A further increase of Re up to 3000 (dominating forced convection regime) promotes the lower cells as shown in Fig. 2c. This promotion occurs to the detriment of the open streamlines that become relatively straight. However, no changes have been noted about flow structures by passing from  = 0 to  = 0.07. The corresponding isotherms show that the addition of 7 % of nanoparticles to the base fluid affects clearly the temperatures distribution. Indeed, the presence of solid particles homogenizes relatively the temperature within the thermally active zone of the cavity.

In the case of the suction mode, the effect of Re on the streamlines and isotherms is presented in Figs. 3a-3c for both pure water and Al2O3-water. As shown in Fig. 3a, the streamlines obtained for low values of Re (Re = 200) show that the flow descends vertically from the upper horizontal inlet and then leaves the cavity horizontally through the vertical two outlets. Consequently, the open lines are parallel to the heated wall allowing more thermal interaction between the fluid and this wall. The structure is characterized too by the presence of two big closed cells surmounting the open streamlines. Such cells are straight and large in size and intensity compared to the upper cells presented in the injection mode (see Fig. 2a). These two cells, having opposite directions of rotation, reflect the considerable shearing effect. Moreover, it can be noticed that the dynamic field is symmetrical with respect to the median of the cavity. For both base fluid and nanofluid, the corresponding isotherms take the form of parallel and dense streamlines at the vicinity of the hot wall; which forms a very limited thermal boundary layer. Consequently, the heat released by the bottom hot wall is quickly and directly transferred to the exit through the thermal boundary layer without ascending under the buoyancy forces. In addition, the cold zone is larger in comparison with the other mode. It results from this behavior that the suction mode is thermally strong. A progressive increase of Re up to 1000 and 3000 [Figs. 3(b)-3(c)] (dominating forced convection regime) leads to a small increase in the size of the two closed cells as a result of the forced flow intensification. The isotherms show that the thermal boundary layer thickness is reduced in favour of the cold zone testifying the growing effect of the forced convection. Finally, it should be underlined that the presence of naoparticles does not promote the change in the flow and thermal structures whatever the value of Re.

Fig. 2: Streamlines and isotherms, in the injection mode, for  = 0 (—) and  = 0.1 (- - -) at different values of Re: a) Re = 500, b) Re = 1000 and c) Re = 3000

a) Re = 500

b) Re = 1000

c) Re = 3000

Fig. 3: Streamlines and isotherms, in the suction mode, for  = 0 (—) and  = 0.1 (- - -) at different values of Re: a) Re = 200, b) Re = 1000 and c) Re = 3000

a) Re = 200

b) Re = 1000

c) Re = 3000

In order to illustrate the performances of injection and suction modes in the heat removal, variations versus Re of the average Nusselt number evaluated along the heated wall, are presented in Fig. 4 for various values of . As expected, this figure shows that globally, an increase of Nu with Re is observed for both injection and suction modes. The rate of this increase becomes more important from Re  500 (an increase in the slope of the curves is observed from this threshold). This tendency is justified by the flow intensification with the inertia effect, promoted by the increase of Re. However, in the injection case, it can be noted that Nu remains almost constant as long as Re < 500. Such behavior is explained by the fact that natural convection dominates for this range of Re (Re < 500) and any increase in Re within this range has a negligible effect on heat transfer. This effect is due to the presence of the two convective cells between the entire heated wall and the open lines and consequently the delaying of heat released by the hot wall towards outside through the open lines. This singularity is absent in the suction mode where the forced flow remains directly in contact with the hot wall what involves a continuous increase in Nu with Re. For a fixed value of Re, the increase of the solid volume fraction ϕ up to 0.07 leads to a noticeable growing effect of the convection in both the injection and suction cases. This is due to the increase in effective thermal conductivity of the nanofluid with the increase in ϕ. More precisely, by increasing ϕ from 0 to 0.07, Nu is increased for the more favorable case, obtained for Re = 3000, from 38.76 / (91.15) to 41.47 / (98.72) for the injection / (suction) mode. Also, it should be noted that, in comparison with the injection mode, the fluid suction mode yields the best heat transfer performance. Quantitatively, for ϕ = 0.07 and Re = 2000 passing from the injection mode to the suction one, Nu increases from 33.68 to 83.66 which corresponds to an enhancement in heat transfer of almost 2.5 times.

For practical reasons, the evaluation of mean temperature of the fluid inside the cavity is of a great importance. Thus, variations of this parameter with Re are plotted in Fig. 5 for both injection and suction modes and different values of . Particularly, in the injection case, the average temperature T increases by increasing Re up to a critical value Rec = 500. This reheating of the cavity is justified by the constant value of the Nusselt number in this range of Re (see Fig.4 for the injection case). Then, this tendency is reversed because the growing of Re is marked by a decrease of T; this behavior is due to the forced convection predominant effect which drives out the heat towards the outside. For the other case (suction), the evolution of mean temperature T with Re, is characterized by a notable decrease with increase in Re, and is due to the stronger convection heat transfer at higher flow velocities. For a fixed value of Re, we can see that the addition of nanoparticles contributes to reduce the average temperature within the cavity for the injection case only. Also, it is interesting to observe that the suction mode leads to a better cooling of the cavity since the resulting values of T are lower than those obtained in the injection mode. For reasons of clarity, it is noted that, for = 0.07 and Re = 1000, a reduction of about 86% of T occurs when passing from the injection mode to the suction one.

Fig. 4: Variations, with Re, of the average Nusselt number, Nu, for both injection and suction modes and different values of 

Fig. 5: Variations, with Re, of the mean temperature, 
, for both injection and suction modes and different values of 

# Conclusion

In the present work, a numerical study was performed to investigate laminar mixed convection of Al2O3-water nanofluid in a horizontal multi-vented enclosure heated from the bottom by a sinusoidal hot temperature. The study was carried out by considering two modes of imposed external flow (injection and suction). From the study, it has been found that the flow and temperature patterns are affected by the ventilation mode. Also, the results illustrate that the suspended nanoparticles substantially increase the heat transfer rate through the cavity and the average temperature within the enclosure by applying both injection and suction modes. In addition to that, it is showed that the suction mode gives a better thermal efficiency and a better cooling effectiveness in comparison with the injection case.

- References

- Khanafer, K., Vafai, K., and Lightstone, M. 2003. Buoyancy-Driven Heat Transfer Enhancement in a Two-Dimensional Enclosure Utilizing Nanofluids. Int. J. Heat Mass Transfer. 46. p. 3639-3653.

- Chandrasekar, M., Suresh, S., and Bose, A. C. 2010. Experimental Investigations and Theoretical Determination of Thermal Conductivity and Viscosity of Al2O3/Water Nanofluid. Experimental Thermal and Fluid Science. 34. p. 210-216.

- Sebdani, S. M., Mahmoodi M., and Hashemi, S. M. 2012. Effect of Nanofluid Variable Properties on Mixed Convection in a Square Cavity. International Journal of Thermal Sciences. 52. p. 112-126.

- Abbasian, A. A., Sebdani, S. M., Mahmoodi, M., Ardeshiri A., and Aliakbari, M. 2012. Numerical Study of Mixed Convection Flow in a Lid-Driven Cavity with Sinusoidal Heating on Sidewalls using Nanofluid. Superlattices and Microstructures. 51(6). p. 893-911.

- Shahi, M., Mahmoudi, A. H., and Talebi, F. 2010. Numerical Study of Mixed Convective Cooling in a Square Cavity Ventilated and Partially Heated from the Below utilizing Nanofluid. International Communications in Heat and Mass Transfer. 37(2). p. 201-213.

- Mahmoudi, A. H., Shahi M., and Talebi, F. 2010. Effect of Inlet and Outlet Location on the Mixed Convective Cooling inside the Ventilated Cavity subjected to an External Nanofluid. International Communications in Heat and Mass Transfer. 37(8). p. 1158-1173.

- Corcione, M. 2011. Empirical Correlating Equations for Predicting the Effective Thermal Conductivity and Dynamic Viscosity of Nanofluids. Energy Conversion and Management. 52(1). p. 789-793.