M. Outanoute1, A. Selmani1, M. Amini Alaoui1, M. Guerbaoui2, A. Ed-dahhak2,

Greenhouse Temperature and Humidity Estimator Design Based on Least Square SVM

A. Lachhab2, B. Bouchikhi1

1 Electronic & Instrumentation Group, Physics department, Faculty of Sciences, 
Moulay Ismaïl University, Meknes, Morocco

2 Modelling, Systems Control and Telecommunications Team, High School of Technology, 
Moulay Ismaïl University, Meknes, Morocco

E-mail: outanoute.med@gmail.com

Abstract- In this paper, a Least Squares Support Vector Machine (LS-SVM), as a modern approach to data analysis and modelling using the methods and tools of artificial intelligence, was employed to model and predict the temperature and relative humidity as the prime climatic parameters for the development and growth of the crop under a greenhouse. In the development of LS-SVM models, linear kernel, polynomial kernel and Gaussian radial basis function (RBF were applied to train the LS-SVM. For that, external climate variables and command inputs were considered as input variables to the model to get the estimated greenhouse temperature and relative humidity. The performances of the LS-SVM models, computed through the Root Mean Square Error (RMSE) and determination coefficient. The proper LS-SVM tuning parameters, the regularization parameter γ and the kernel function parameter σ were adjusted to get the best LSSVM outputs. The simulation results showed that RBF kernel boasted better precision and generalization than linear and polynomial kernels and thus able to provide precision for the relevant greenhouse climate parameters estimation.

# Keywords: Greenhouse climate, Modelling, Regression, Estimation, Least Squares Support Vector Machine (LSSVM).

# INTRODUCTION

As is well known, research on greenhouse climate control has gained much interest during the last years in the field of agriculture; as it allows the creation of an indoor microclimate for crop development, protecting it from adverse outdoor conditions in order to increase quality and quantity of greenhouse production and also permits to cultivate certain plants all over the year [1-2]. Therefore, this microclimate is controlled by artiﬁcial actuation such as heating and ventilation using effective control techniques to provide the best optimum environmental growing conditions for the crops [3-4]. However, for plants of high complexity, like greenhouse process, it is of major importance to developing as accurate models as possible. On the basis of a precise model, one can then design a reliable simulator that makes it possible to define the modes of the plant functioning under different operating conditions that are significant for the tuning of the control system and for the plant supervision [5-6]. One common approach to estimate greenhouse climate model is to develop data-driven models validated with measured data that relate the variables (input-output relationship) in the system. On the other hand, this extraction of knowledge from the data requires large datasets and sophisticated modelling techniques [7]. Various methods are used to model and to predict the temperature and relative humidity in many greenhouses systems [8]. Support vector machine (SVM) is a supervised learning model with a learning algorithm that becomes more popular technique which analysed data and recognized patterns that are used for classification and regression analysis [9-10].

In this work, the support vector regression based prediction will be used to model and to predict the temperature and relative humidity under greenhouse and compare the performance between three different type of LSSVM kernels models namely; RBF, Linear and polynomial kernels.

# Material and Methodology

## Experimental Setup

The experimental greenhouse used in this experimentation is shown in Fig. 1. Is equipped with a control system that allows both the acquisition and monitoring and automatic control of greenhouse climate parameters. The internal climate is defined by the internal temperature; the internal relative humidity and CO2 content, while the external is composed of the Temperature, Relative humidity, and solar radiation, all of these are measured with its corresponding sensors. For that, the signals delivered from sensors are conditioned, amplified and then transmitted to the computer system through the broad data acquisition PCI 6024E device from National Instrument, the control and operating software was designed in the Matlab/Simulink environment to implement the real-time control. The system is able to provide a real-time monitoring of different climatic parameters inside and outside the greenhouse. In fact, the system was used as support for developing and testing various advanced control strategies [11-13].

- Schematic diagram of greenhouse experimental system.

## Data Collection and Preprocessing

For modelling purposes, this experiment collected data of three days, acquired with a sampling time of 10 seconds, contain outside and inside climate variables measured were internal temperature (Tint), external temperature (Text), internal relative humidity (RHint) and external relative humidity (RHext), and the data pattern of input excitation signals sequences which are Heater (cmdH) and ventilator (cmdV) actuators command corresponds to the outputs variables (temperature and relative humidity), were shown in Fig. 2.

- Data input and output variables (a) Command heater, (b) Command ventilator, (c) External temperature, (d) External relative humidity, (e) Internal temperature and (f) Internal relative humidity.

The original data collected are not in the same order of magnitude, which can influence the accuracy of the neural network training. For that all data must be pre-treated and normalized as follows:

(1)

where x is the original data, xn the normalized input or output values, xmin and xmax are the minimum and maximum values of the concerned variable respectively.

## LS-SVM Modelling Theory

Least Square Support Vector Machine (LS-SVM) is a supervised learning model with a learning algorithm [14]. They offer a very powerful method of deriving efficient models for multidimensional linear and nonlinear classification and regression analysis [15]. By a nonlinear function φ, the data are mapped from the original feature space to a higher dimensional transformed one (Fig. 3).

- An LS-SVM neural network structure.

The nonlinear function φ is approximate in a linear way as follows [16]:

(2)

with the given training data set where N represents the number of training data set, xi is the input data, and yi is the output data, w is the weight vector, φ(.) is the nonlinear mapping function and b is the bias.

The optimal regression function is given according to the structural minimization principle as [17]:

(5)

Subject to the equality constraints:

(6)

where ei is the residual error variable.

In order to solve the optimization problem, a Lagrangian function is defined as follows [18]:

(7)

where αi are the Lagrange multipliers which can be either positive or negative values. The conditions for optimality can be obtained by solving the partial derivatives from the objective function with respect to the parameters which yields the linear regression of [19]:

(8)

where α=[α1, α2, …, αN]T is called the support vector, whose support values αi are proportional to the residual errors ei by the regularization factor γ ∈ R and I is an identity matrix.

Besides, the kernel trick is employed to derive that:

(9)

where K is a predefined kernel function.

Then, the solution for the LS-SVM regression problem can be derived [20]:

(10)

where K is the kernel function satisfying Mercer’s condition, xi is the training data, and x denotes the new input data.

For the LS-SVM modelling, both γ and σ parameters should be determined for each input data set. For this purpose in this study γ and σ were determined through a trial-error process.

There are several choices of kernel function available which are known as follow [21]:

- Linear kernel

(11)

- Polynomial kernel

𝑡 ≥ 0, 					(12)

with t the intercept and d the degree of the polynomial.

- Radial Basis Function (RBF) kernel

(13)

with 𝜎2 the variance of the Gaussian kernel.

The LS-SVM model was developed with three different SVM kernel functions (linear, polynomial and RBF). The performances of these three different LS-SVM models were evaluated. For each type of SVM model developed, few different kernel functions were applied in order to analyse the best kernel and parameter that will give the best result. The LS-SVM tuning parameters sigma (γ) and gamma (σ) were manually adjusted to get the best LS-SVM output models.

## Statistical Performance Indices

In this study, two different performance criteria of this model were evaluated based on the difference value between the real system outputs data and the predicted value from the model. Namely, the Mean Square Error (MSE), the Variance Accounted For (VAF). The calculation was performed using equations (14) and (15):

(14)

(15)

where N is the number of training or testing data pairs, y(k) and  are, respectively, the measured value and estimated model outputs value.

# Modelling Results

The LS-SVM modelling algorithm is applied to analyse its performance in the greenhouse temperature and relative humidity estimation model. For that three models with different kernel functions (linear, polynomial and RBF) are compared. Each of the SVM models was trained with the training data and the resulted output was recorded. For that, the data of the two first days were used for the training purpose of the LS-SVM model. After running for train data, the LS-SVM model was being verified with test data set in order to ensure the accuracy of the system and to verify the capability in predicting the best result. The analysis has been carried out on the trained LS-SVM model by varying each of kernels type and its parameters Different values of γ and σ have been tried to get the best result. Training and testing analysis of LS-SVM have been carried out by using MATLAB.

The structure diagram of greenhouse climate parameters estimator based on LSSVM is as Fig. 4.

- LSSVM-based greenhouse climate parameters estimator structure diagram.

The estimation results of greenhouse temperature humidity system are shown in Fig. 5, Fig. 6 and Fig. 7 for the three models based on linear, polynomial and RBF kernel respectively.

It is seen that form this comparison results that the trained LS-SVM model with the RBF kernel captures the greenhouse temperature and relative humidity accurately than the models elaborated by using the linear and polynomial kernels. Therefore, the results demonstrate a good generalization capacity of the LSSVM greenhouse model.

- : Mesaured and estimated outputs by Linear kernel LSSVM model.

- Mesaured and estimated outputs obtained by polynomial kernel LSSVM model.

- Mesaured and estimated outputs obtained by RBF kernel LSSVM model.

The resulted RMSE and VAF performance indices, evaluated between measured and estimated temperature and relative humidity determined by different models used in this study are tabulated in Table 1 and Table 2.

Table I 
Training RMSE indices for all LS-SVM models

| Methods | Training RMSE |  |
| --- | --- | --- |
|  | Tint | RHint |
| Linear kernel | 0.0290 | 0.0423 |
| Polynomial kernel | 0.0155 | 0.0189 |
| RBF kernel | 0.0148 | 0.0172 |

Table II: 
Test MSE and VAF indices for all LS-SVM models

| Methods | Test RMSE |  | Test VAF (%) |  |
| --- | --- | --- | --- | --- |
|  | Tint | RHint | Tint | RHint |
| RBF kernel | 0.083 | 0.160 | 99.56 | 99.12 |
| Polynomial kernel | 0.970 | 1.328 | 98.52 | 97.93 |
| Linear kernel | 1.133 | 1.454 | 95.86 | 95.08 |

From the training and testing results, it shows that RBF kernel LS-SVM gives the best performance with the lowest root mean squared error and a better VAF value as compared to polynomial and linear kernel LS-SVM model. Therefore, the superiority of the LS-SVM with kernel function model over the models is verified for the temperature and relative humidity estimation task.

# Conclusion

In this study, three different SVM models were applied to estimate and to predict the two prime climatic parameters under greenhouse system (temperature and humidity). It can be concluded that RBF kernel LSSVM produced the most outstanding result as compared to the linear and polynomial LSSVM models. It was also found that RBF kernel function made a better kernel function because it predicts better than the linear and polynomial kernel. Finally, LSSVM is promising technique to use for estimation of the climate parameter under greenhouse system.

- References

- Shamshiri R., Ismail W. I. W. A review of greenhouse climate control and automation systems in tropical regions. Journal of Agricultural Science and Applications. 2013. 2(3). p. 176-183.

- Van Beveren P. J. M., Bontsema J., van Straten G., and Van Henten E. J. Optimal control of greenhouse climate using minimal energy and grower defined bounds Applied Energy. 2015. 159. p. 509-519.

- Lu H. G., Li C. Y., and Jiang J. P. Application of intelligence control in agriculture greenhouses. In Applied Mechanics and Materials. 2015. 719. p. 293-297.

- Van Beveren, P.J.M., Bontsema, J., van Straten, G., and van Henten, E.J. Minimal heating and cooling in a modern rose greenhouse. Appl. Energy. 2015. 137. p. 97-109.

- Ou, Y., Yi, H. and Jiang, L. Greenhouse temperature variation study. Northern Horticulture. 2014. 4. p. 37-41.

- Zou W., Yao F., Zhang B., He C., and Guan Z. Verification and predicting temperature and humidity in a solar greenhouse based on convex bidirectional extreme learning machine algorithm. Neurocomputing. 2017. 249. p. 72-85.

- Suzuki Y., Kaneda Y., and Mineno H. Analysis of Support Vector Regression Model for Micrometeorological Data Prediction. Computer Science and Information Technology. 2015. 3(2). p. 37-48.

- Samui P., Sitharam T. G. Application of Least Squares Support Vector Machine in seismic attenuation prediction. ISET Journal of Earthquake Technology. 2009. 46 (3-4). p. 147-155.

- Samui P. Application of Least Square Support Vector Machine (LSSVM) for determination of evaporation losses in reservoirs. Engineering. 2011. 3(4). p. 431-434.

- Sachindra D. A., Huang F., Barton A., and Perera B. J. C. Least square support vector and multi-linear regression for statistically downscaling general circulation model outputs to catchment streamflows. International Journal of Climatology. 2013. 33(5). p. 1087-1106.

- Ed-Dahhak A., Guerbaoui M., El Afou Y., Outanoute M., Lachhab A., Belkoura L., and Bouchikhi B. Implementation of fuzzy controller to reduce water irrigation in greenhouse using LabVIEW. International Journal of Engineering and Advanced Technology Studies. 2013. 1(2). p. 12-22.

- El Afou Y., Belkoura L., Outanoute M., Guerbaoui M., Rahali A., Ed-Dahhak A., Lachhab A., Join C., and Bouchikhi B. Feedback techniques using PID and PI intelligent for greenhouse temperature control. International Journal of Advanced Research in Electrical, Electronics and Instrumentation Engineering. 2014. 3(6). p. 9779-9792.

- Guerbaoui M., El Afou Y., Ed-dahhak A., Lachhab A. and Bouchikhi B. PC-based automated drip irrigation system. International Journal of Engineering Science and Technology. 2013. 5(1). p. 221-225.

- Stephen J. E., Kumar S. S., and Jayakumar J. Nonlinear modeling of a switched reluctance motor using LSSVM-ABC. Acta Polytechnica Hungarica. 2014. 11(6). p. 143-158.

- Yu H., Chen Y, Hassan S. G., and Li D. Prediction of the temperature in a Chinese solar greenhouse based on LSSVM optimized by improved PSO. Computers and Electronics in Agriculture. 2016. 122. p. 94-102.

- Balabin R. M., Lomakina E. I. Support vector machine regression (SVR/LS-SVM)-an alternative to neural networks (ANN) for analytical chemistry. Comparison of nonlinear methods on near infrared (NIR) spectroscopy data. Analyst. 2011. 136(8). p. 1703-1712.

- Liao, J., Qu, B., and Zheng, N. Effects of process parameters on the extraction of quercetin and rutin from the stalks of euonymus alatus (thumb) sieb and predictive model based on least squares support vector machine optimized by an improved fruit fly optimization algorithm. Applied Sciences. 2016. 6(11). p. 340-359.

- Ramedani Z., Omid M., Keyhani A., Khoshnevisan B., and Saboohi H. A comparative study between fuzzy linear regression and support vector regression for global solar radiation prediction in Iran. Solar Energy. 2014. 109. p. 135-143.

- Sun F., Li X., Liao H., and Zhang, X. A bayesian least-squares support vector machine method for predicting the remaining useful life of a microwave component. Advances in Mechanical Engineering. 2017. 9(1). p. 1-9.

- Ahmadi, M. A. Toward reliable model for prediction drilling fluid density at wellbore conditions: A LSSVM model. Neurocomputing. 2016. 211. p. 143-149.

- Gorjaei, R. G., Songolzadeh, R., Torkaman, M., Safari, M., & Zargar, G. A novel PSO-LSSVM model for predicting liquid rate of two phase flow through wellhead chokes. Journal of Natural Gas Science and Engineering, 2015. 24. p. 228-237.